Step by Step

- STM32CubeMX New Project.

- Board Selector:

  - Vendor:

    - STMicroelectronics

  - Type of Board:

    - Discovery

  - MCU Series:

    - STM32F4

  - Board List:

    - STM32F4Discovery

- OK

- Pinout:

  - Middlewares:

    - FreeRTOS

  - Peripherals:

    - ADC1

    - ADC2

    - ADC3

    - CAN1

    - CAN2

    - CRC

    - I2C1 – I2C

    - I2S3 – Full-Duplex Master

    - RCC – High Speed Clock (HSE) Crystal/Ceramic
RCC – Low Speed Clock (LSE) Disable

    - RNG

    - SPI1 – Full-Duplex Master

    - TIM1 – Internal Clock

    - TIM2 – Internal Clock

    - USB_OTG_FS – OTG/Dual_Role_Device

- Configuration:

  - IS23 – Multimedia – 49 kHz

- Project:

  - Settings

    - Project Name:

      - FreeRTOS

    - Toolchain/IDE

      - SW4STM32

    - Generated Files – Generate peripheral initialization as a pair …

  - OK

- Project:

  - Generate Code

- Close STM32CubeMX

- In Windows Explorer

  - Delete Configuration from the folder name as the space will cause a failure
“FreeRTOS\SW4STM32\FreeRTOS Configuration”

- Edit .cproject

  - Remove Configuration from both occurrences of artifactName

    - <configuration artifactExtension="elf" artifactName="FreeRTOS Configuration"

    - <configuration artifactExtension="elf" artifactName="FreeRTOS"

- Edit .project

  - Remove Configuration from name

    - <name>FreeRTOS Configuration</name>

    - <name>FreeRTOS</name>

- Copy STM32Cube\Repository\STM32Cube_FW_F4_V1.5.0\Drivers\BSP to FreeRTOS\Drivers

- In FreeRTOS\Drivers\BSP, delete all content except for, Components and STM32F4-Discovery

- Copy STM32Cube\Repository\STM32Cube_FW_F4_V1.5.0\Middlewares\ST to
FreeRTOS\Middlewares

- Delete STemWin

- The on disk directory tree should look like this

- Start System Workbench

  - Import Existing Projects into Workspace

  - Choose the workspace used by System Workbench

  - Select FreeRTOS and click Finish

  - Project Properties (ALT + Enter)

    - Resource:

      - Text File Encoding – UTF-8

    - C/C++ Build

      - Settings

        - Tool Settings

          - Floating point hardware – fpv4-sp-d16

          - Floating point ABI – hard

        - Build Steps (add this to generate bin and hex files)

          - Post Build Steps

            - Command:
arm-none-eabi-objcopy -O binary "${BuildArtifactFileBaseName}.elf" "${BuildArtifactFileBaseName}.bin"
&
arm-none-eabi-objcopy -O ihex "${BuildArtifactFileBaseName}.elf" "${BuildArtifactFileBaseName}.hex"

        - Target:

          - Board: STM32F4Discovery

    - Click on OK

  - Select the Project and choose Build All, result is two warnings

  - Edit main.c, compare my file to your main.c, mine works

  - Add the BSP folder to the Project

    - Click on Drivers, then Import from File System, Navigate to BSP and select Components and STM32F4-Discovery and then click Finish

  - Clean the Project and then Build All

  - Click on Run, Debug Configurations, click on AC6 STM32 Debugging and click on New

  - Click on the Debug button

  - After the code is loaded into the STM32F4 Discovery Board, click on the Resume ICON.

At this point, you should have a working FreeRTOS.

Main.c

/**

******************************************************************************

* File Name          : main.c

* Date               : 03/05/2015 21:49:06

* Description        : Main program body

******************************************************************************

*

* COPYRIGHT(c) 2015 STMicroelectronics

*

* Redistribution and use in source and binary forms, with or without modification,

* are permitted provided that the following conditions are met:

*   1. Redistributions of source code must retain the above copyright notice,

*      this list of conditions and the following disclaimer.

*   2. Redistributions in binary form must reproduce the above copyright notice,

*      this list of conditions and the following disclaimer in the documentation

*      and/or other materials provided with the distribution.

*   3. Neither the name of STMicroelectronics nor the names of its contributors

*      may be used to endorse or promote products derived from this software

*      without specific prior written permission.

*

* THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS "AS IS"

* AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE

* IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE ARE

* DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT HOLDER OR CONTRIBUTORS BE LIABLE

* FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL

* DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR

* SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER

* CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY,

* OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE

* OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

*

******************************************************************************

*/

/* Includes ------------------------------------------------------------------*/

#include "../Drivers/BSP/STM32F4-Discovery/stm32f4_discovery.h"

#include "stm32f4xx_hal.h"

#include "cmsis_os.h"

#include "adc.h"

#include "can.h"

#include "crc.h"

#include "i2c.h"

#include "i2s.h"

#include "rng.h"

#include "spi.h"

#include "usb_otg.h"

#include "gpio.h"

/* USER CODE BEGIN Includes */

/* USER CODE END Includes */

/* Private variables ---------------------------------------------------------*/

/* USER CODE BEGIN PV */

/* LED Private Variables */

osThreadId LEDThread1Handle, LEDThread2Handle, LEDThread3Handle,

LEDThread4Handle;

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

void MX_FREERTOS_Init(void);

/* USER CODE BEGIN PFP */

/*LED Function Prototypes */

static void LED_Thread1(void const *argument);

static void LED_Thread2(void const *argument);

static void LED_Thread3(void const *argument);

static void LED_Thread4(void const *argument);

/* USER CODE END PFP */

/* USER CODE BEGIN 0 */

/* USER CODE END 0 */

int main(void) {

/* USER CODE BEGIN 1 */

/* USER CODE END 1 */

/* MCU Configuration----------------------------------------------------------*/

/* Reset of all peripherals, Initializes the Flash interface and the Systick. */

HAL_Init();

/* Configure the system clock */

SystemClock_Config();

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_ADC1_Init();

MX_ADC2_Init();

MX_ADC3_Init();

MX_CAN1_Init();

MX_CAN2_Init();

MX_CRC_Init();

MX_I2C1_Init();

MX_I2S3_Init();

MX_RNG_Init();

MX_SPI1_Init();

MX_USB_OTG_FS_USB_Init();

/* USER CODE BEGIN 2 */

/* Configure all four LEDs*/

BSP_LED_Init(LED3);

BSP_LED_Init(LED4);

BSP_LED_Init(LED5);

BSP_LED_Init(LED6);

/* Thread 1 definition and assign to LED */

osThreadDef(LED3, LED_Thread1, osPriorityNormal, 0,

configMINIMAL_STACK_SIZE);

/* Thread 2 definition and assign to LED */

osThreadDef(LED4, LED_Thread2, osPriorityNormal, 0,

configMINIMAL_STACK_SIZE);

/* Thread 3 definition and assign to LED */

osThreadDef(LED5, LED_Thread3, osPriorityNormal, 0,

configMINIMAL_STACK_SIZE);

/* Thread 4 definition and assign to LED */

osThreadDef(LED6, LED_Thread4, osPriorityNormal, 0,

configMINIMAL_STACK_SIZE);

/* Start Thread 1 */

LEDThread1Handle = osThreadCreate(osThread(LED3), NULL);

/* Start Thread 2 */

LEDThread2Handle = osThreadCreate(osThread(LED4), NULL);

/* Start Thread 3 */

LEDThread3Handle = osThreadCreate(osThread(LED5), NULL);

/* Start Thread 4 */

LEDThread4Handle = osThreadCreate(osThread(LED6), NULL);

/* USER CODE END 2 */

/* Call init function for freertos objects (in freertos.c) */

MX_FREERTOS_Init();

/* Start scheduler */

osKernelStart();

/* We should never get here as control is now taken by the scheduler */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1) {

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

}

/* USER CODE END 3 */

}

/* Toggle LED3 */

static void LED_Thread1(void const *argument) {

uint32_t count = 0;

(void) argument;

for (;;) {

count = osKernelSysTick() + 5000;

/* Toggle LED3 every 200 ms for 5 s */

while (count >= osKernelSysTick()) {

BSP_LED_Toggle(LED3);

osDelay(200);

}

/* Turn off LED3 */

BSP_LED_Off(LED3);

/* Suspend Thread 1 */

osThreadSuspend(NULL);

count = osKernelSysTick() + 5000;

/* Toggle LED3 every 400 ms for 5 s */

while (count >= osKernelSysTick()) {

BSP_LED_Toggle(LED3);

osDelay(400);

}

/* Resume Thread 2 */

osThreadResume(LEDThread2Handle);

}

}

/* Toggle LED4 */

static void LED_Thread2(void const *argument) {

uint32_t count;

(void) argument;

for (;;) {

count = osKernelSysTick() + 10000;

/* Toggle LED4 every 500 ms for 10 s */

while (count >= osKernelSysTick()) {

BSP_LED_Toggle(LED4);

osDelay(500);

}

/* Turn off LED4 */

BSP_LED_Off(LED4);

/* Resume Thread 1 */

osThreadResume(LEDThread1Handle);

/* Suspend Thread 2 */

osThreadSuspend(NULL);

}

}

/* Toggle LED5 */

static void LED_Thread3(void const *argument) {

uint32_t count;

(void) argument;

for (;;) {

count = osKernelSysTick() + 10000;

/* Toggle LED5 every 500 ms for 10 s */

while (count >= osKernelSysTick()) {

BSP_LED_Toggle(LED5);

osDelay(500);

}

/* Turn off LED5 */

BSP_LED_Off(LED5);

/* Resume Thread 4 */

osThreadResume(LEDThread4Handle);

/* Suspend Thread 3 */

osThreadSuspend(NULL);

}

}

/* Toggle LED6 */

static void LED_Thread4(void const *argument) {

uint32_t count;

(void) argument;

for (;;) {

count = osKernelSysTick() + 10000;

/* Toggle LED6 every 500 ms for 10 s */

while (count >= osKernelSysTick()) {

BSP_LED_Toggle(LED6);

osDelay(500);

}

/* Turn off LED6 */

BSP_LED_Off(LED6);

/* Resume Thread 3 */

osThreadResume(LEDThread3Handle);

/* Suspend Thread 4 */

osThreadSuspend(NULL);

}

}

/** System Clock Configuration

*/

void SystemClock_Config(void) {

RCC_OscInitTypeDef RCC_OscInitStruct;

RCC_ClkInitTypeDef RCC_ClkInitStruct;

RCC_PeriphCLKInitTypeDef PeriphClkInitStruct;

__PWR_CLK_ENABLE()

;

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE;

RCC_OscInitStruct.HSEState = RCC_HSE_ON;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE;

RCC_OscInitStruct.PLL.PLLM = 8;

RCC_OscInitStruct.PLL.PLLN = 336;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 7;

HAL_RCC_OscConfig(&RCC_OscInitStruct);

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_SYSCLK | RCC_CLOCKTYPE_PCLK1

| RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5);

PeriphClkInitStruct.PeriphClockSelection = RCC_PERIPHCLK_I2S;

PeriphClkInitStruct.PLLI2S.PLLI2SN = 192;

PeriphClkInitStruct.PLLI2S.PLLI2SR = 2;

HAL_RCCEx_PeriphCLKConfig(&PeriphClkInitStruct);

}

/* USER CODE BEGIN 4 */

/* USER CODE END 4 */

#ifdef USE_FULL_ASSERT

/**

* @brief Reports the name of the source file and the source line number

* where the assert_param error has occurred.

* @param file: pointer to the source file name

* @param line: assert_param error line source number

* @retval None

*/

void assert_failed(uint8_t* file, uint32_t line)

{

/* USER CODE BEGIN 6 */

/* User can add his own implementation to report the file name and line number,

ex: printf("Wrong parameters value: file %s on line %d\r\n", file, line) */

/* USER CODE END 6 */

}

#endif

/**

* @}

*/

/**

* @}

*/

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/